SYSTEM001
GLIDER GNC

Technical Report — Algorithm Pipeline & Architecture

CAN-7U-SAT Parafoil Guided Recovery System

Project: Rocket Team — System001
Date: 2026-06-22
Status: Pre-Training Verification Complete

# Table of Contents

1. Mission Overview

2. System Architecture

2.1 Hardware Stack

2.2 Software Module Map

3. Algorithm Pipeline — Full Data Flow

4. Detailed Subsystem Descriptions

4.1 State Estimation (Madgwick, EKF, RLS Wind)

4.2 State Machine (6-Phase FSM)

4.3 Guidance — PID Controller

4.4 Guidance — RL Controller (SAC)

4.5 Actuator Mixing

4.6 Physics Simulation

5. Evaluation System

6. Current Performance

7. Bugs Found & Fixed

8. Improvement Plan

9. Key Design Decisions

10. File Reference

# 1. Mission Overview

System001 is a precision autonomous parafoil recovery system for a sounding rocket (CAN-7U-SAT). After rocket motor burnout and apogee, a drogue parachute deploys to slow descent. At 600m AGL, a parafoil (ram-air canopy) deploys and the onboard Guidance, Navigation & Control (GNC) computer autonomously steers the glider back to the launch pad.

Goal: Land within 15 meters of the launch pad (CEP90 ≤ 15m) under winds up to 8 m/s.

## Mission Phases

| Phase | Description |
| --- | --- |
| 1. Rocket Boost | 50N thrust for 3 seconds, then ballistic coast to apogee |
| 2. Apogee Detection | Barometer detects 5m altitude drop + vertical velocity < -2 m/s |
| 3. Drogue Descent | Drogue chute deployed, terminal velocity ~10 m/s, heavy wind drift |
| 4. Parafoil Deployment | 10-sample baro moving average ≤ 600m AGL triggers canopy release |
| 5. Guided Descent | Active GNC control — PID or RL steers toward launch pad |
| 6. Flare & Landing | Symmetric brake at <10m altitude, servos neutral after touchdown |

# 2. System Architecture

## 2.1 Hardware Stack (Target: Raspberry Pi)

| Component | Part | Interface | Data |
| --- | --- | --- | --- |
| IMU | ICM-20948 | I²C (0x69) | 9-DOF: Accel (m/s²), Gyro (rad/s), Mag (µT) |
| Barometer | BMP388 | I²C (0x77) | Pressure (Pa), Temperature (°C), Altitude (m) |
| GPS | u-blox | UART (9600) | Lat, Lon, Alt, Ground Speed, Heading, Fix |
| Current Sensor | INA219 | I²C (0x40) | Voltage (V), Current (A) |
| Servos | 2× (Left/Right Brake) | PWM | Range: 60°–120°, Neutral: 90° |
| Telemetry | XBee | Serial | Downlink packets to ground station |

## 2.2 Software Module Map

| Module | Purpose |
| --- | --- |
| config/gains.yaml | All tunable parameters in one YAML file |
| sensors/drivers.py | IMUData, BaroData, GPSData dataclasses + I²C/UART stubs |
| hw_interface/base.py | Abstract HWInterface class (read_imu, read_baro, write_servos, etc.) |
| hw_interface/simulated_hardware.py | SITL implementation — reads directly from dynamics model |
| estimation/madgwick.py | Madgwick AHRS — 9-DOF quaternion sensor fusion |
| estimation/ekf_altitude.py | Extended Kalman Filter for altitude + vertical velocity |
| estimation/wind_estimator.py | Recursive Least Squares wind estimator |
| state_machine/flight_states.py | 6-state finite state machine |
| guidance/heading_pid.py | PID heading controller with anti-windup |
| guidance/altitude_budget.py | S-turn energy management logic |
| guidance/rl_guidance.py | SAC/DQN policy loader & inference wrapper |
| training/env.py | Gymnasium environment (12-dim obs, 2-dim continuous action) |
| training/train_sac.py | SAC training loop with 3-stage curriculum + buffer flush |
| sim/dynamics.py | 3-phase physics engine (Rocket, Drogue, Glider) |
| sim/wind_model.py | Randomizable wind model ("blowing toward" convention) |
| sim/scenario_validator.py | Reachability filter for Monte Carlo scenarios |
| sim/monte_carlo.py | 500-drop statistical evaluation engine |
| sim/run_sim.py | Narada Dashboard — real-time 3D visualization |
| flight_computer.py | Main 20 Hz flight loop — runs on real hardware or SITL |

# 3. Algorithm Pipeline — Full Data Flow

The entire system runs at 20 Hz (50ms loop). Below is the complete signal path from raw sensor readings to servo actuation, executed every single frame:

## Stage 1: Sensor Input

IMU (ICM-20948) — 9-axis: accel_x/y/z (m/s²), gyro_p/q/r (rad/s), mag_x/y/z (µT)

Barometer (BMP388) — Pressure (Pa), Temperature (°C) → Altitude (m) via standard atmosphere

GPS (u-blox) — Lat, Lon, Altitude, Ground Speed, Ground Track Heading, Fix status

## Stage 2: State Estimation

Madgwick AHRS — Fuses all 9 IMU channels into a quaternion, outputs roll/pitch/yaw in radians

Frame Transform — Body-to-earth Z acceleration transform: accel_z_earth = -sin(θ)·ax + sin(φ)·cos(θ)·ay + cos(φ)·cos(θ)·az

EKF Altitude — Predict step uses earth-frame vertical acceleration; Update fuses baro (R=2.0) and GPS altitude (R=5.0)

RLS Wind Estimator — Uses GPS ground velocity components + compass heading to solve V_ground = Va·[cos ψ, sin ψ] + [Vw_x, Vw_y]

## Stage 3: State Machine

The FSM receives fused altitude and vertical velocity from the EKF. It transitions through: BOOST → DROGUE_DESCENT → DEPLOYMENT_TRIGGER → DEPLOYMENT_VERIFICATION → GUIDED_DESCENT → LANDED. Active GNC control only runs during GUIDED_DESCENT.

## Stage 4: Guidance (PID Path)

Wind Compensation — Compute aim point: target - (Vw × time_to_target), where time_to_target = altitude / sink_rate

Target Heading — target_heading = atan2(aim_y - current_y, aim_x - current_x)

Heading Error — Heading error with ±π wrap: e = (target - current + π) mod 2π - π

Gain Scheduling — Gains change by altitude: Cruise (>200m, gentle), Approach (50-200m, moderate), Final (<50m, aggressive)

PID Computation — δa = Kp·e + Ki·∫e·dt + Kd·de/dt, clamped to ±30°

Flare Logic — 30° symmetric brake when altitude < 10m, else 0°

## Stage 4: Guidance (RL Path)

Observation — 12-element vector: [sin(heading_err), cos(heading_err), dist/1000, alt_excess/1000, wind_speed/10, sin(wind_dir), cos(wind_dir), pitch/0.5, roll/0.5, yaw_rate/0.5, prev_δa/30, prev_δs/30]

Policy Inference — SAC neural network (2-layer MLP, 256 units) outputs [δa, δs] directly

## Stage 5: Control Output

Servo Mixing — left_servo = 90° + δs - δa; right_servo = 90° + δs + δa

Clamping — Both channels clamped to [60°, 120°] physical limits

# 4. Detailed Subsystem Descriptions

## 4.1 State Estimation

### Madgwick AHRS Filter

Input: 9-DOF IMU (accelerometer, gyroscope, magnetometer)

Output: Quaternion → Euler angles (roll, pitch, yaw) in radians

Algorithm: Gradient descent optimization on the orientation quaternion. The gyroscope provides high-frequency integration while the accelerometer and magnetometer provide low-frequency correction via a computed gradient.

Key Parameter: β = 0.1 (controls how aggressively the accel/mag correct the gyro drift)

Why Madgwick: Better convergence with magnetometer data than Mahony; proven in embedded systems; single tuning parameter.

### Extended Kalman Filter — Altitude

State Vector: x = [altitude, vertical_velocity]ᵀ

Predict Step: x_k = F·x_{k-1} + B·u, where u = vertical acceleration from IMU (earth frame)

Update Sources: Barometer (R = 2.0, lower noise) and GPS altitude (R = 5.0, higher noise)

Process Noise: Q = diag(0.01, 0.1) — process noise representing unmodeled vertical dynamics

Kalman Gain: Standard Kalman gain: K = P·Hᵀ·(H·P·Hᵀ + R)⁻¹; State update: x = x + K·(z - H·x)

### RLS Wind Estimator

Model: V_ground = Va·[cos ψ, sin ψ] + [Vw_x, Vw_y]

State Vector: θ = [Va, Vw_x, Vw_y]ᵀ — jointly estimates airspeed and 2D wind vector

Algorithm: Recursive Least Squares with forgetting factor λ = 0.98

Excitation: A 5-second sinusoidal heading wobble is injected above 100m AGL to provide persistent excitation for observability. Wobble fades linearly to zero below 100m for precise final approach.

## 4.2 State Machine (6-Phase FSM)

| State | Entry Condition | Action |
| --- | --- | --- |
| BOOST | Launch detected | Servos locked neutral; record launch position |
| DROGUE_DESCENT | Baro dropped 5m + Vz < -2 m/s | Deploy drogue chute |
| DEPLOYMENT_TRIGGER | 10-sample baro avg ≤ 600m AGL + descending | Fire parafoil release pyro |
| DEPLOYMENT_VERIFICATION | Immediate transition | Verify canopy inflation (placeholder) |
| GUIDED_DESCENT | Canopy verified | Active GNC control enabled |
| LANDED | AGL ≤ 5m | Servos neutral; log final miss distance |

## 4.3 PID Heading Controller with Gain Scheduling

The PID controller computes the asymmetric brake command (δa) to steer the parafoil. The heading error is computed with proper ±π wrapping to avoid discontinuity. Anti-windup limits the integral to ±(output_limit / Ki).

### Gain Schedule

| Phase | Altitude | Kp | Ki | Kd | Rationale |
| --- | --- | --- | --- | --- | --- |
| Cruise | > 200m | 5.0 | 0.05 | 0.5 | Gentle — plenty of altitude remaining |
| Approach | 50–200m | 10.0 | 0.1 | 1.0 | Moderate — tighten tracking |
| Final | < 50m | 20.0 | 0.2 | 2.0 | Aggressive — last chance to correct heading |

[NOTE] Gain scheduling prevents control saturation: at high altitude, gentle corrections avoid oscillation; near the ground, aggressive corrections maximize terminal accuracy.

## 4.4 RL Controller — Soft Actor-Critic (SAC)

### Why SAC?

Soft Actor-Critic is chosen because: (1) it is off-policy, making it sample-efficient — important when each simulated flight takes hundreds of timesteps; (2) it natively supports continuous action spaces, which matches the parafoil's 2-axis brake system; (3) entropy regularization encourages exploration, preventing premature convergence to local optima in the wind-disturbed environment.

### Observation Space (12 dimensions)

| Index | Variable | Normalization | Source |
| --- | --- | --- | --- |
| 0 | sin(heading_error) | [-1, 1] | Guidance geometry |
| 1 | cos(heading_error) | [-1, 1] | Guidance geometry |
| 2 | distance_to_target | ÷ 1000 | Guidance geometry |
| 3 | altitude_excess | ÷ 1000 | (alt - dist/glide_ratio) |
| 4 | wind_speed | ÷ 10 | Wind model |
| 5 | sin(wind_direction) | [-1, 1] | Wind model |
| 6 | cos(wind_direction) | [-1, 1] | Wind model |
| 7 | pitch | ÷ 0.5 rad | Dynamics model |
| 8 | roll | ÷ 0.5 rad | Dynamics model |
| 9 | yaw_rate | ÷ 0.5 rad/s | Dynamics model |
| 10 | prev_delta_a | ÷ 30° | Temporal context |
| 11 | prev_delta_s | ÷ 30° | Temporal context |

[NOTE] Heading error and wind direction use sin/cos encoding to eliminate the ±180° wrap discontinuity that would confuse gradient-based learning. Attitude rates (pitch, roll, yaw_rate) were added after a verification pass identified their absence as a root cause for poor control authority.

### Action Space (2 continuous)

| Action | Range | Meaning |
| --- | --- | --- |
| δa (asymmetric brake) | [-30°, +30°] | Turn left / turn right |
| δs (symmetric brake) | [0°, 30°] | Slow down / flare for landing |

### Reward Function

reward = (prev_distance - current_distance)

- 0.05 × Σ|action_t - action_{t-1}|

+ { +10000 if miss < 10m,

+2000  if miss < 20m,

+200   if miss < 50m }  (at landing only)

Term 1: Potential-based shaping (prev_dist - curr_dist) gives dense signal without reward hacking

Term 2: Action smoothness penalty covers both δa AND δs, prevents servo oscillation

Term 3: Sparse terminal bonuses give massive signal at landing to drive final precision

### Curriculum Learning (3 Stages)

| Stage | Progress | Wind (m/s) | Drop Range (m) | Airspeed (m/s) | Glide Ratio |
| --- | --- | --- | --- | --- | --- |
| 1 (Easy) | 0–20% | 0–2 | 200–500 | 12 (fixed) | 4.5 (fixed) |
| 2 (Medium) | 20–50% | 0–4 | 300–800 | 11–14 | 4.0–5.0 |
| 3 (Full) | 50–100% | 0–8 | 500–1500 | 10–18 | 3.0–6.0 |

[NOTE] Replay Buffer Management: At each stage transition, 80% of the replay buffer is flushed. This prevents old easy-stage transitions from dominating gradient updates when the environment becomes harder. Without this fix, the 1M-transition buffer was drowning out Stage 3 learning signal.

## 4.5 Actuator Mixing

Both PID and RL output two commands:

δa — asymmetric brake (turn command)

δs — symmetric brake (speed/flare command)

Mixing equations:

left_servo  = 90° + δs - δa

right_servo = 90° + δs + δa

Both clamped to [60°, 120°]

## 4.6 Physics Simulation (3-Phase Dynamics)

### Phase 1 — Rocket

50N thrust for 3 seconds, then ballistic coast. Wind drift applied.

### Phase 2 — Drogue

First-order convergence to -10 m/s terminal velocity. Heavy wind drift, no control authority.

### Phase 3 — Glider (Parafoil)

Turn Dynamics: Asymmetric brake → commanded yaw rate via K_turn = 0.5/30 (30° brake ≈ 0.5 rad/s turn rate), with first-order lag (τ = 0.2s)

Airspeed: Base 12 m/s, reduced by symmetric brake (max -2 m/s at full brake)

Glide Ratio: Base 4.0, reduced by symmetric brake (max -1.0 at full brake)

Ground Velocity: V_ground = V_air × [cos ψ, sin ψ] + [Vw_x, Vw_y]

Sink Rate: V_sink = V_air / (L/D)

Wind Model: Constant-direction, constant-speed per scenario. Mathematical "blowing toward" convention. Randomized 0–8 m/s for Monte Carlo.

# 5. Evaluation System

## 5.1 Monte Carlo Simulation

500 drops per evaluation run

Fixed random seed (42) ensures identical scenario lists between PID and SAC runs

Domain randomization: Drop distance (500–1500m), wind (0–8 m/s), airspeed (10–18 m/s), glide ratio (3.0–6.0)

Both PID and SAC evaluations use CPU inference for fair timing comparison

## 5.2 Reachability Filter

Before simulating each drop, the filter checks physical feasibility:

max_still_air_range = altitude × glide_ratio

tailwind_component  = wind_speed × cos(wind_dir - bearing_to_target)

effective_groundspeed = airspeed + tailwind_component

if effective_groundspeed ≤ 0 → UNREACHABLE (headwind exceeds airspeed)

if max_effective_range < distance × 1.3 → UNREACHABLE (too far for glide ratio)

[NOTE] The 1.3× safety margin accounts for turn losses and imperfect tracking. All 21 unreachable scenarios from a 500-drop run were manually spot-checked — every one has a genuine positive shortfall.

## 5.3 Metrics

| Metric | Definition |
| --- | --- |
| CEP50 | Median miss distance (50th percentile) |
| CEP90 | 90th percentile miss distance — PRIMARY TARGET (goal: ≤ 15m) |
| Success Rate | Percentage of drops landing within 20m of target |

# 6. Current Performance

## PID Baseline (with gain scheduling + corrected filter)

| Metric | Value |
| --- | --- |
| Drops Attempted | 500 |
| Unreachable Filtered | 20 |
| Reachable Evaluated | 480 |
| Success Rate (< 20m) | 2.5% |
| CEP50 (Median Miss) | 36.94 m |
| CEP90 (90th %) | 156.25 m |
| Max Miss Distance | 838.80 m |

## SAC (100k steps — severely undertrained)

| Metric | Value |
| --- | --- |
| Drops Attempted | 500 |
| Unreachable Filtered | 21 |
| Reachable Evaluated | 479 |
| Success Rate (< 20m) | 3.3% |
| CEP50 (Median Miss) | 576.01 m |
| CEP90 (90th %) | 1990.74 m |
| Max Miss Distance | 3705.23 m |

[IMPORTANT] The SAC numbers reflect a severely undertrained policy (only 100k steps with the old 9-dim observation). This is expected "passive drift" behavior — the agent has barely begun to learn. The observation space expansion and replay buffer fixes address the root causes preventing effective learning.

# 7. Bugs Found & Fixed During Verification

| # | Bug | Impact | Fix Applied |
| --- | --- | --- | --- |
| 1 | Reachability filter inverted — subtracted tailwind instead of headwind | Filtered out easy scenarios, kept impossible ones; made PID appear worse | Corrected to airspeed + tailwind_component |
| 2 | Observation space missing attitude rates — no pitch, roll, or yaw_rate | Agent had zero kinematic feedback; couldn't anticipate turn dynamics | Expanded obs from 9 → 12 dimensions |
| 3 | Replay buffer not flushed at curriculum transitions — 1M buffer retained all easy-stage data | Stage 3 gradient updates dominated by calm-wind/short-distance transitions from Stages 1-2 | 80% buffer flush at each stage advance |
| 4 | Monte Carlo miss distance read from obs index — obs[2] × 1000 hardcoded | Would silently break when obs array shape changed | Replaced with direct math.hypot(dynamics.x, dynamics.y) |

# 8. Improvement Plan — What's Next

## Phase 1: Rolling-Average Curriculum (Pending Approval)

Replace fixed step-percentage stage advancement with a success-rate-based trigger. Advance from Stage N to N+1 when the trailing 50-episode success rate (miss < 50m) exceeds 60%. This prevents premature advancement before the agent has learned the current difficulty.

## Phase 2: Extended Training Run (10M Steps)

With the corrected 12-dim observation, buffer flush, and rolling-average curriculum. Checkpoint every 500k steps with automatic Monte Carlo evaluation. Target: CEP90 ≤ 15m on the reachable subset.

## Phase 3: Sim-to-Real Transfer

Replace SimulatedHardware with real I²C/UART drivers on Raspberry Pi

Add sensor noise injection during training for domain-randomized robustness

Hardware-in-the-loop testing with actual servos and parafoil canopy

# 9. Key Design Decisions

| Decision | Rationale |
| --- | --- |
| Madgwick over Complementary filter | Better mag fusion, proven in embedded systems, single tuning parameter |
| EKF for altitude (not full-state) | 2-state EKF is fast enough for 20 Hz on RPi; full 15-state EKF unnecessary for parafoil |
| RLS for wind (not EKF) | Simpler, fewer states, explicit forgetting factor for time-varying wind |
| SAC over PPO | Off-policy (sample efficient) + continuous actions match the 2-axis brake |
| Potential-based reward over raw distance | Prevents "distance trap" where agent gets stuck at local minima |
| sin/cos angle encoding | Eliminates wrap discontinuity at ±180° that breaks gradient learning |
| Gain scheduling (3 phases) | Gentle corrections at altitude (time to correct), aggressive at ground level (last chance) |
| Curriculum learning (3 stages) | Full domain randomization from step 0 makes learning intractable |
| Buffer flush at stage transitions | Without it, easy-stage data overwhelms hard-stage learning signal |

# 10. File Reference

| File | Lines | Purpose |
| --- | --- | --- |
| flight_computer.py | 199 | Main 20 Hz loop — real or SITL |
| sim/dynamics.py | 130 | 3-phase physics engine |
| estimation/madgwick.py | 100 | Quaternion AHRS filter |
| estimation/ekf_altitude.py | 80 | Kalman altitude fusion |
| estimation/wind_estimator.py | 76 | RLS wind estimator |
| guidance/heading_pid.py | 36 | PID with anti-windup |
| guidance/altitude_budget.py | 48 | S-turn energy management |
| guidance/rl_guidance.py | 77 | RL policy loader |
| training/env.py | 150 | Gymnasium training environment |
| training/train_sac.py | 90 | SAC training + curriculum + buffer flush |
| state_machine/flight_states.py | 72 | 6-state FSM |
| sim/monte_carlo.py | 157 | 500-drop evaluation |
| sim/scenario_validator.py | 29 | Reachability filter |
| config/gains.yaml | 68 | All tunable parameters |